SCOPE OF WORK

Laboratory will conduct PFAS Analyses (29 analytes) in accordance with applicable EPA Methods 533 and 537.1 to acceptable minimum reporting levels (MRL) consistent with the requirements of UCMR5.

EPA METHOD 537.1

EPA METHOD 533

EPA METHOD 533 (continued)

In addition to the Sample Analyses, the Laboratory shall provide miscellaneous items and services to include the following:  EDD – Standard, Field Blank Extraction & Analyses*, Hardcopy Reports (via PDF), QC Level II, Sample Kits, Delivery of Sample Kits, Sample Disposition/Disposal, and Data Tracking & Management.

*Note:  PFAS Field Blanks will be extracted and analyzed if the associated sample has detections greater than the MRL.

Client will be responsible for sample collection and delivery to the laboratory in acceptable condition.  Laboratory to provide coolers and all associated containers, with shipping costs to Hawaii included in the bid offer.  Shipment costs back to the laboratory shall be borne by the client.  Sample disposal shall be inclusive with the bid offer.

ADDITIONAL INSTRUCTIONS

Laboratory must be approved by EPA to Support PFAS Analyses under UCMR5 and be certified for analysis of PFAS in drinking water by the State of Hawaii.  Lithium testing will not be required.

The Project tests allotment is as follows:

- 60 sample tests per EPA Method 533

- 8 blank test per EPA Method 533

- 60 sample tests per EPA Method 537.1

- 8 blank tests per EPA Method 537.1

The laboratory shall provide notice, to the State project manager, of any PFAS detection in the initial screening sample within 48 hours of testing.  A positive result will trigger a second “confirmation” sample to be taken from the available balance.  The notification to the State shall be made via e-mail to the State Project Manager

PROJECT TIMEFRAME:

START DATE:                     		June 1,2023

SAMPLE COLLECTION:     		June 1,2023 to June 23, 2023

FINAL LAB ANALYSES REPORTS:    	July 28, 2023

END DATE:       				August 25, 2023   (including final invoicing by lab)