STATE OF HAWAII
DEPARTMENT OF TRANSPORTATION
AIRPORT DIVISION - OAHU DISTRICT

Daniel K. Inouye International Airport

Rescue Watercraft (RIB Boat)

APRIL 26, 2023

Scope of Work

Provide a quotation for the following good/service/construction:

Rigid Inflatable Boat. Boat construction must be of fiberglass hull construction with DTX Hypalon material air tubes. All items shall meet the following requirements and specifications listed below in relation to the product requested.

Specifications of Vessel

Bow Length Overall (LOA): 15 feet 7 inches.

Internal Length: 11 feet 10 inches

Beam: 6 feet 11 inches

Internal Beam: 4 feet 1 inches

Draft (without Outboard Engine): 12 inches

Rated Minimum, Adult Passengers (SOB): 6

Weight (w/o engine): 650 lbs. - 850 lbs.

Fuel Tank:  Aluminum, 16-gallon, internal fuel tank with filter and gauge

Chambers: 5 ea.

Pressure Relief Valves: 5 ea.

Tube Diameter: 18 inches

Tube Material: 1670 DTX HYPALON, with non-marking, high profile, high density, heavy duty rub strake with two (2) boarding wear patches, minimum 1/8 inch thick, black in color. Heavy duty external lifelines, mounted outside of tube (no lacing cuffs). Seems shall be triple bonded, overlapped and reinforced with internal and external seam tape.

Rub Strake: Minimum profile of 1.25 inches off the tube and a minimum overall height of 5 inches.

Tube Color:  Orange

Hull:  Solid Fiberglass (FRP) hull, with internal liner. Longitudinal and transvers composite stringer system

Hull Construction: Inorganic materials only, “No Wood”

Hull Color: Orange gelcoat equipped with a keel guard.

Vessel Deck: Heavy Duty Industrial molded non-skid with removable panel for below deck complete fuel tank access.

High-Capacity Deck Drains: Minimum of 3.5” diameter and have external drain trunks that can be secured to prevent water flow.

Electrical Wiring: All wiring, terminals, circuit breakers shall meet or exceed the American Boat & Yacht Council (ABYC) Standards for Electrical Practice. The wiring shall be marine grade with soldered terminals and heat shrink terminal covers, suitable for operation in a saltwater environment. Marine grade circuit breakers shall be provided for all circuits.

Equipment Provided:

- Fiberglass (FRP) forward position console with attached pod seating, with hinged locking storage and high support stainless steel cushioned backrest.

- Stainless steel seat console grab handle

- One /two-person bench seat with storage with hinging cover to access underneath.

- Access Hatch

- Cable steering

- Anchor locker

- Bow eye.

- Bow cleat.

- Two stern tie-down eyes

- Four switch electrical panel

- Battery

- Battery switch

- Navigation lights

- Antenna arch (aluminum material) integrated with mooring tie-off points and dive ladder (6061 aluminum material). Ladder to be hinging and able to be stowed in the upright position. Ladder to have a minimum of three steps, all with non-slip surface.

- Garmin ECHOMAP UHD 74sv GPSMAP Sounder and installation (Removable)

- 2 - Led flood lights (Pair)

- 12v Outlet

- Spotlight, with remote control

- Tow-Post located on the bow of the vessel. Bright Anodized, reinforced 6061 Aluminum with a diameter of 2.5”

- Tow-Post located on the aft of the vessel. Bright Anodized, Reinforced 6061 Aluminum with a diameter of 2.5”, with aft support arms.

- Compass

- Electrical bilge pump with high-capacity deck drains

- Lifelines

- Foot pump

- Load Rite heavy duty galvanized bunk trailer, with single axle, with jack

- Operational and maintenance manuals

- Manufacture Certificate of Origin for Vessel AND Trailer or Manufacture Statement of Origin.

Engine Provided

- Suzuki 70HP Four Stroke with aluminum propeller, to include tachometer.

- Complete rigging and complete installation

- Complete gauge package

- Prop Guard, installed.

Operational and maintenance manuals

Guarantee, Standards & Warranty

The offer shall guarantee that the items provided shall be clean, new, unused (no demonstration, sample, or display models), of the current production year and model. The items shall be in good working condition and free from defects in material, workmanship, and operation, and of the highest quality that is acceptable to the requesting agency. The construction and craftsmanship of the respective item shall be performed in accordance with the best standard industrial practice used in the manufacture of similar items. All items shall fit, match, and/or be compatible with the other items being offered without any alterations, modifications, or additional costs.

Product must be free from defects in materials and/or faulty workmanship for a period of no more than 1 year from the date of delivery. The warranty shall be limited to the repair or replacement, at bidder’s option, if components shown to be defective in either workmanship or materials.

Brand or Trade Name

Whenever a brand or manufacturer's name, designation or material appears in connection with the specified item, such name or designation indicates the standard of quality desired, and the reference shall follow applicable trademark or Copyright laws.

The Officer-In-Charge (O-I-C)

State of Hawaii, Department of Transportation - Airports Division, Airports Fire Chief or his designate.

Payments

Payment will be paid for all items delivered in whole. There will be no partial payments rendered for items delivered in portions.

Delivery/Received Dates

All items will be delivered in whole (one shipment) and delivery address within 180 days from the date notification (letter to proceed) is received by the vendor.  Freight shall be Container Load and Block.

Delivery Address:

Daniel K. Inouye International Airport

Airport Fire Station #2.

31 Lagoon Drive

Honolulu, Hawaii 96819

If the entirety of order is not received within 180 days, the Airports Fire Chief reserves the absolute right to rescind purchase from selected vendor and award bid to the next lower bidder.  Any delivery extensions must be approved by the Airport Fire Chief or his designate.

Configuration and description of goods and service

The written price quote must be noted to include the items total cost, to include shipping and tax to the requested location.