ATTACHMENT A

TOPOGRAPHIC SURVEY REQUIREMENTS FOR RECONFIGURATION & RESURFACING OF PARKING LOT IN HIARNG 282/1898

Honolulu, HI

- The surveyor shall research available public records for all mapping, monumentation, plats, government surveys, etc. that may pertain to the subject property and shall research all applicable public utilities for substructure data such as sewers, storm drains, water lines, electrical conduits, etc.

- The survey shall be performed on the ground in accordance with the current “Accuracy Standards for Land Title Surveys” as adopted by the American Congress on Surveying and Mapping, the National Society of Professional Surveyors, and the American Land Title Association.

- On the plat or map, the survey boundary shall be drawn to a convenient scale, with the scale clearly indicated. A graphic scale, shown in feet, shall be included. A north arrow shall be shown on the plat or map of the survey and shall be oriented so that north is at the top of the drawing. Symbols or abbreviations used shall be identified on the face of the plat or map by use of a legend or other means. Supplementary or exaggerated diagrams shall be presented accurately on the plat or map where dimensional data is too small to be shown clearly at full scale.

- The surveyor shall place a 3-1/2” diameter bronze survey marker cap, stamped or engraved with the following information:

  - “STATE OF HAWAII DEPARTMENT OF DEFENSE, HAWAII NATIONAL GUARD”

  - Survey station ID in 5 alphanumeric characters (eg. HI001)

  - Horizontal control in North American Datum (NAD83) HARN (decimal degrees).

  - Vertical control in Local Tidal Datum (LTD) in feet and meters.

  - Triangular surveying center mark with center dimple.

  - “SURVEY MARKER”

  - “DO NOT DISTURB”

- The survey shall contain the following applicable information:

  - Locate all utilities inside and outside the tunnel.

  - The name, address, telephone number, and signature of the Professional Land Surveyor who made the survey, the surveyor’s official seal and registration number in good standing with the State of Hawaii, the date the survey was completed and dates of all revisions.

  - The survey drawing(s) submitted shall bear the following certification adjacent to the Engineer’s official seal:  “I hereby certify that all information indicated on this drawing was obtained or verified by actual measurements in the field and that every effort has been made to furnish complete and accurate information.”

  - Vicinity map showing the property in reference to nearby highways or major street intersections.

  - Flood zone designation (with proper annotation based on Federal Flood Insurance Rate Maps).

  - Land area as defined by the boundaries of the legal description of the surveyed premises.

  - All data necessary to indicate the mathematical dimensions and relationships of the boundary represented by bearings and distances, and the length and radius of each curve together, together with elements necessary to mathematically define each curve. The point of beginning of the surveyor’s description and the basis of bearings shall also be shown.

  - When record bearings or angles or distances differ from measured bearings, angles or distances, both record and measured bearings, angles, and distances shall be clearly indicated. If the record description fails to form a mathematically closed figure, the surveyor shall so indicate.

  - Measured and record distances from corners of parcels surveyed to the nearest right-of-way lines of streets in urban or suburban areas, together with recovered lot corners and evidence of lot corners, shall be noted. The distances to the nearest intersecting street shall be indicated and verified. Names and widths of streets and highways abutting the property surveyed and widths of right–of-ways shall be given. Observable evidence of private roads shall be so indicated. Streets abutting the premises, which have been described in Record Documents, but not physically opened, shall be shown and so indicated.

  - The identifying titles of all recorded plats, filed maps, right of way maps, or similar documents which the survey represents, wholly or in part, with their appropriate recording data. The survey shall indicate the platted setback or building restriction lines which have been recorded in subdivision plats or which appear in a Record Document which has been delivered to the surveyor. Contiguity, gores, and overlaps along the exterior boundaries of the survey premises, where ascertainable from field evidence or Record Documents, or noted. Where only part of a recorded lot or parcel is included in the survey, the balance of the lot or parcel shall be indicated.

  - All evidence of found monuments shall be shown and noted. All monuments found beyond the surveyed premises on which establishment of corners of the survey premises are dependent, and their application related to the survey shall be indicated.

  - The character of any and all evidence of possession shall be stated, and the location of such evidence carefully given in relation to both the measured boundary lines and those established by the record. An absence of notation on the survey shall be presumptive of no observable evidence of possession. The term “possession” does not imply “ownership”.

  - The location of all buildings upon the plot or parcel shall be shown and their locations defined by measurements perpendicular to boundaries. If there are no buildings, so state. Proper street numbers shall be shown where available.

  - All easements evidenced by a Record Document which have been delivered to the surveyor shall be shown, both those burdening and those benefitting the property surveyed, indicating recording information. If such an easement cannot be located, a note to this affect shall be included. Observable evidence of easements and/or servitudes of all kinds, such as those created by roads, rights-of-ways, water courses, drains, telephone, telegraph, or electric lines, water, sewer, oil or gas pipelines on or across the surveyed property and on adjoining properties if they appear to affect the surveyed property, shall be located and noted. Surface indications, if any, and other visible improvements within five feet of each side of the boundary lines shall be noted. Without expressing a legal opinion, physical evidence of all encroaching structural appurtenances and projections by or adjoining property or abutting streets, on any easement or over setback lines shown by Record Documents shall be indicated with the extent of such encroachment or projections.

  - Driveways and alleys on or crossing the property must be shown. Where there is evidence of use by other than the occupants of the property, the surveyor must also indicate on the plat or map. Where driveways or alleys on adjoining properties encroach, in whole or in part, on the property being surveyed, the surveyor must so indicate on the plat or map with appropriate measurements.

  - Location, alignment and dimensions of all paved and unpaved roads, curbs, walks, parking and paved areas abutting the subject land. Indicate road centerlines with true bearings and lengths by 50 foot stationing. Describe curves by designating the points of curvature and tangency by station. Include all curve data as well a location of radius and vertex points. Elevations on 50 foot centers on centerlines of roads, edges of roads and top and bottom of curbs.

  - As accurately as the evidence permits, the location of cemeteries and burial grounds disclosed in the process of researching title to the premises or observed in the process of performing the field work for the survey, shall be shown.

  - Pond, lakes, wetlands, springs, or streams bordering on or running through the premises being surveyed shall be shown. When a property surveyed contains a natural water boundary, the surveyor shall measure the location of the boundary according to appropriate surveying methods and note on the plat or map the date of measurement and the caveat that the boundary is subject to change due to natural causes and that it may or may not represent the actual location of the limit of the title. When the surveyor is aware of changes in such boundaries, the extent of those changes shall be identified.

  - Topographic contours are to be surveyed at 1 foot intervals. Base vertical control on the permanent (not assumed) National Geodetic Survey (NGS). Note location, description, and datum.

  - Exterior dimensions of all buildings at ground level. Show square footage of exterior footprint of all buildings at ground level.\

  - Elevations at each entrance to buildings, service docks building corners, steps, ramps, and grade slabs.

  - Substantial, visible improvements (in additions to buildings) such as signs, parking areas, concrete pads, etc.

  - Parking areas and, if striped, the striping and the type (e.g., Accessible, motorcycle, regular, etc.) and number of parking spaces.

  - Indication of access to a public way such as curb cuts and driveways.

  - Observable evidence of frequently used dirt or gravel trails (e.g., walkways & roads).

  - Location of utilities existing on or serving the surveyed property as determined by observed evidence together with plans and markings provided by utility companies, and other appropriate sources (with references as to the source of information. Locate and show all fire hydrants located within 500 feet of the subject property.

  - Manholes, catch basins, valve vaults or other surface indications of subterranean uses.

  - Leach fields, absorption beds, cesspools and septic tanks as shown in Record Drawings and by observed evidence.

  - Drainage basins, swales, channels, and other surface storm water systems.

  - Wires cables, (including their function), crossing the survey premises, and all poles on or within ten feet of the surveyed premises, and the dimensions of all cross-wires or overhangs affecting the surveyed premises.

  - Utility company installations on the surveyed premises.

  - Names of the adjacent owners of the platted lands.

  - Observable evidence of earth moving work, building construction, or building additions within recent months.

  - Any changes in street right-of-way lines either completed or proposed, and available from the controlling jurisdiction. Observable evidence of recent street or sidewalk construction or repairs.

  - Observable evidence of site use as a solid waste dump, sump, or sanitary landfill.

  - All trees with a minimum diameter of 6” measured at 48” above the base of the tree. Perimeter outline only of thickly wooded areas with description of predominant vegetation.

  - Locations of archeological sites of historical or potentially historical significance as evidenced by observation or Record Documentation.