# INVITATION FOR BIDS (IFB) NO. 26-4679

# FURNISH AND DELIVER VEHICLES

# FOR FLEET SERVICES

# UNIVERSITY OF HAWAII HONOLULU, HAWAII

JANUARY 2026

BOARD OF REGENTS UNIVERSITY OF HAWAII HONOLULU, HAWAII

# TABLE OF CONTENTS

IFB No. 26-4679	to Furnish and Deliver Vehicles for Fleet Services, University of Hawaii, Honolulu, Hawaii

| NOTICE TO BIDDERS | 3 |
| --- | --- |
| BID REQUIREMENTS | 4 |
| TECHNICAL SPECIFICATIONS | 5 |
| GENERAL AND SPECIAL PROVISIONS | 9 |

IT IS THE RESPONSIBILITY OF ALL BIDDERS TO CHECK THE TABLE OF CONTENTS TO CONFIRM THAT ALL PAGES LISTED THEREIN ARE CONTAINED IN THEIR BID PACKAGE.

## NOTICE TO BIDDERS

The University of Hawaii IFB No. 26-4679, Vehicles for Fleet Services, University of Hawaii, Honolulu, Hawaii is issued and will be awarded through the State of Hawaii’s electronic procurement system (HIePRO).

All bid responses must be submitted electronically through HIePRO no later than 2:30 p.m., February 26, 2026, and at that time will be publicly opened. Bids received after the due date and time or received in a form other than electronically through HIePRO will not be considered.

Bidders are advised that they should not wait until the last minute to submit their bid through HIePRO. Bidders are solely responsible for ensuring that their electronic submission through HIePRO is complete and all necessary files (Mandatory Bid Form) are attached to their bid prior to the IFB due date and time. The University shall not be responsible for any delay or failure of any Bidder to submit any materials updated through the IFB process on a timely basis.

Electronic Procurement

Bidders interested in responding to this electronic solicitation must be registered on HIePRO. To register, visit the following link: https://hiepro.ehawaii.gov/videos/video/vendor_registration.html. Reference the Vendor Quick Reference Guide for additional information at https://hiepro.ehawaii.gov/static-resources/VendorQuickReferenceGuide.pdf.

HIePRO will be the system of record for the issuance of the IFB, to receive the Mandatory Bid Form and other Bid requirements, issue Amendments, and make award for the IFB. Amendments and other information and materials provided through HIePRO, may include additions or changes with respect to the due date and time.

Special instructions in HIePRO related to this solicitation are incorporated herein and made a part of this IFB through reference. Bidders shall review all special instructions located in HIePRO.

Questions and Clarifications

All questions and requests for clarifications must be submitted electronically through HIePRO. Questions must be submitted by January 29, 2026 at 2:00 p.m., Hawaii Standard Time. Responses will be posted on February 12, 2026.  The University may refuse to answer any questions received outside of HIePRO or after the Questions/Answers deadline.

Luis P Salaveria

Chief Procurement Officer

University of Hawaii

Posting Date: January 15, 2026

Vendors downloading the IFB shall be responsible for notifying the Procurement Specialist Bonnie Anderson (e-mail: bonnie27@hawaii.edu), so that the name, address, phone number, fax number, and e-mail address of the vendor can be listed on the University’s register for the purpose of notification of any amendments to the IFB which are issued.

# BID REQUIREMENTS

FURNISH AND DELIVER VEHICLES FOR FLEET SERVICES

UNIVERSITY OF HAWAII HONOLULU, HAWAII

By attaching The Mandatory Bid Form to HIePRO, the bidder has carefully examined the INVITATION FOR BIDS (IFB) NO. 26-4679, TO FURNISH AND DELIVER VEHICLES FOR FLEET SERVICES, UNIVERSITY OF HAWAII,  and offers to furnish and deliver the vehicles for as required by the University in strict accordance with the true intent and meaning of the Invitation for Bids (IFB), and shall complete delivery within One-Hundred and Twenty (120) consecutive calendar days from the date designated in the Notice to Proceed, as follows:

TAX LIABILITY

Both out-of-state and Hawaii bidders are advised that the amount bid on this solicitation is subject to the general excise tax imposed by Chapter 237, Hawaii Revised Statutes (HRS) and, if tangible property is being imported into the State of Hawaii for resale, the use tax (currently 1/2%) imposed by Chapter 238, HRS.  (Refer to Taxes in the General Provisions.)  Bidders are therefore cautioned to consider such taxes in formulating their bids since no adjustments to the prices bid shall be allowed.

BASIS FOR AWARD

The award of contract, if awarded, shall be made to the lowest responsive and responsible bidder on the BID PRICE FOR EACH ITEM (1-6). Bidders do not need to bid on all items to be considered for an award.

NOTE TO BIDDERS

An acceptable bid must conform in all material respects to this Invitation for Bids.  Any of the following may be grounds for disqualification:

1.	Taking exception to any of the specifications, terms or conditions contained in the IFB.

2.	Placing conditions on the furnishing of solicited goods or services.

3.	Inclusion of a quotation or order form containing additional specifications, terms or conditions.

4.	Referencing external documents containing additional specifications, terms or conditions.

Bidders are advised that bids are evaluated as submitted and requests by bidders to delete conditions contained in their bids after bid opening cannot be considered.

## TECHNICAL SPECIFICATIONS

This section indicates the Technical Specifications for the vehicles required. The Technical Specifications listed herein are the minimum requirements and are mandatory for an accepted bid.

The technical specifications listed herein define the minimum acceptable requirements. The Bidder shall be responsible for furnishing and delivering vehicles that meet or exceed all the mandatory specifications.

Compliance: Bidders must be able to meet the specifications exactly as written unless a deviation is explicitly noted.

Deviations: Bidders must clearly identify and list all deviations from the specifications on the Mandatory Bid Form or an attached schedule. Failure to list deviations may result in the bid being rejected as non-responsive. The University reserves the right to accept or reject any deviations.

All questions pertaining to the Technical Specifications shall be must be submitted electronically through HIePRO. Questions must be submitted by January 29, 2026 at 2:00 p.m., Hawaii Standard Time. Responses will be posted on February 12, 2026.  -

Bidders are cautioned to review the Technical Specifications carefully and thoroughly. Objections to or requests for clarification of the specifications shall be made in writing in accordance with the General Provisions to the Office of Procurement and Real Property Management prior to the submittal of a bid. The submittal of a bid shall be considered as acceptance of the specifications as published.

### ITEM 1. FIVE (5) New (2024 or later) Electric Four-Door Multi-Purpose Vehicles

| Category | Mandatory Specifications |
| --- | --- |
| Manufacturer and Model | Manufacturer’s Standard |
| Condition & Year | New, 2024 or later model year |
| Vehicle Type | Compact electric multi-purpose vehicle (MPV) |
| Color | White exterior color; interior color is not specified |
| Body Style | Four-door sedan or crossover with standard manufacturer roof height |
| Drivetrain | No drivetrain preference; any manufacturer-standard configuration (FWD, RWD, AWD, or equivalent) is acceptable |
| Transmission | Automatic transmission, any manufacturer-standard configuration |
| Performance | Vehicle must be suitable for standard electric MPVH fleet operations. Specific battery capacity, range, motor output, cargo volume, or charging specifications are not required |
| Safety Features | Standard manufacturer-provided safety features necessary for safe operation |
| Infotainment & Connectivity | Standard manufacturer-provided infotainment and connectivity features |
| Interior Features | Standard manufacturer-provided interior features necessary for safe operation. Keyless entry is required |
| Warranty | Maximum care extended warranty, 7 years/75,000 miles. Zero (0) deductible. (or equivalent) |
| Compliance Standards | Compliant with FMVSS, EPA, and CARB Zero-Emission Vehicle (ZEV) standards |

### ITEM 2. ONE (1) Electric or Hybrid 7-Passenger Van (New, 2024 or later)

| Category | Mandatory Specifications |
| --- | --- |
| Manufacturer and Model | Manufacturer’s Standard |
| Condition & Year | New, 2024 or later model year |
| Vehicle Type | Electric or Plug-in Hybrid 7-Passenger Van |
| Color | White exterior color. Interior color is not specified |
| Seating Capacity | Minimum 7 passengers (2 front, 2nd and 3rd rows) |
| Drivetrain | No drivetrain preference; any manufacturer-standard configuration (FWD, RWD, AWD, or equivalent) is acceptable |
| Powertrain | Electric or Hybrid Powertrain with regenerative braking |
| Transmission | Automatic transmission, any manufacturer-standard configuration |
| Performance | Vehicle must be suitable for standard 7-passenger van fleet operations. Specific battery capacity, range, motor output, cargo volume, or charging specifications are not required |
| Safety Features | Standard manufacturer-provided safety features necessary for safe operation |
| Interior Features | Standard manufacturer-provided interior features necessary for safe operation |
| Warranty | Maximum care extended warranty, 7 years/75,000 miles. Zero (0) deductible. (or equivalent) |
| Compliance Standards | FMVSS and EPA/CARB standards for zero- or low-emission vehicles |

### ITEM 3. ONE (1) Electric 1/2 Ton 4-Door Pickup Truck (New, 2024 or later)

| Category | Mandatory Specifications |
| --- | --- |
| Manufacturer and Model | Manufacturer’s Standard |
| Condition & Year | New, 2024 or later model year |
| Vehicle Type | Electric Pickup Truck (BEV) |
| Color | White exterior color. Interior color is not specified |
| Seating Capacity | Minimum 4 passengers |
| Drivetrain | No drivetrain preference; any manufacturer-standard configuration (FWD, RWD, AWD, or equivalent) is acceptable |
| Transmission | Automatic transmission, any manufacturer-standard configuration |
| Performance | Vehicle must be suitable for standard 1/2 ton electric pickup truck fleet operations. Specific bed length, battery capacity, range, motor output, charging, towing, and payload specifications are not required |
| Safety Features | Standard manufacturer-provided safety features necessary for safe operation |
| Interior Features | Standard manufacturer-provided interior features necessary for safe operation |
| Warranty | Maximum care extended warranty, 7 years/75,000 miles. Zero (0) deductible. (or equivalent) |
| Compliance Standards | FMVSS, EPA, CARB ZEV compliant |

### ITEM 4. ONE (1) Electric Cargo Van (New, 2024 or later)

| Category | Mandatory Specifications |
| --- | --- |
| Manufacturer and Model | Manufacturer’s Standard |
| Condition & Year | New, 2024 or later model year |
| Vehicle Type | Electric Cargo Van (BEV) |
| Color | White exterior color. Interior color is not specified |
| Seating Capacity | Minimum 1 passenger (driver) |
| Drivetrain | No drivetrain preference; any manufacturer-standard configuration (FWD, RWD, AWD, or equivalent) is acceptable |
| Transmission | Automatic transmission, any manufacturer-standard configuration |
| Performance | Vehicle must be suitable for standard electric cargo van fleet operations. Specific battery capacity, range, motor output, cargo volume, payload, and charging specifications are not required |
| Safety Features | Standard manufacturer-provided safety features necessary for safe operation |
| Interior Features | Standard manufacturer-provided interior features necessary for safe operation |
| Warranty | Maximum care extended warranty, 7 years/75,000 miles. Zero (0) deductible. (or equivalent) |
| Compliance Standards | FMVSS, OSHA vehicle safety, and CARB ZEV standards |

### ITEM 5. ONE (1) Hybrid Compact Pickup Truck (New, 2024 or later)

| Category | Mandatory Specifications |
| --- | --- |
| Manufacturer and Model | Manufacturer’s Standard |
| Condition & Year | New, 2024 or later model year |
| Vehicle Type | Hybrid Compact Pickup Truck |
| Color | White exterior color. Interior color is not specified |
| Seating Capacity | Minimum 2 passengers |
| Drivetrain | No drivetrain preference; any manufacturer-standard configuration (FWD, RWD, AWD, or equivalent) is acceptable |
| Powertrain | Gasoline-Electric Hybrid |
| Performance | Vehicle must be suitable for standard hybrid compact pickup truck fleet operations. Specific fuel economy, towing, and payload specifications are not required |
| Transmission | Automatic transmission, any manufacturer-standard configuration |
| Safety Features | Standard manufacturer-provided safety features necessary for safe operation |
| Interior Features | Standard manufacturer-provided interior features necessary for safe operation |
| Warranty | Maximum care extended warranty, 7 years/75,000 miles. Zero (0) deductible. (or equivalent) |
| Compliance Standards | FMVSS and EPA emissions compliance |

### ITEM 6. TWO (2) 3/4 Ton Pickup Trucks (New, 2024 or later)

| Category | Mandatory Specifications |
| --- | --- |
| Manufacturer and Model | Manufacturer’s Standard |
| Condition & Year | New, 2024 or later model year |
| Vehicle Type | 3/4 Ton Pickup Truck |
| Color | White exterior color. Interior color is not specified |
| Seating Capacity | Minimum 2 passengers |
| Drivetrain | Four-Wheel Drive |
| Engine | Gasoline Engine, minimum 6.6L or equivalent |
| Transmission | Automatic transmission, any manufacturer-standard configuration |
| Towing Package | Standard manufacturer-installed towing package appropriate for a 3/4 ton work truck |
| Performance | Vehicle must be suitable for standard 3/4 ton fleet operations. Specific horsepower, towing, and payload specifications are not required |
| Safety Features | Standard manufacturer-provided safety features necessary for safe operation |
| Interior Features | Standard manufacturer-provided interior features necessary for safe operation |
| Warranty | Maximum care extended warranty, 7 years/75,000 miles. Zero (0) deductible. (or equivalent) |
| Compliance Standards | FMVSS and EPA emissions compliant |

## GENERAL AND SPECIAL PROVISIONS

- SCOPE

The Furnishing and Delivery of Vehicles for Fleet Services, University of Hawaii shall be in accordance with the terms and conditions of IFB No. 26-4679 and the General Provisions dated September 2013 included by reference. Copies of the General Provisions are available at the Office of Procurement and Real Property Management, University of Hawaii, 1400 Lower Campus Road, Room 15, Honolulu, Hawaii 96822 or the General Provisions may be viewed at: https://www.hawaii.edu/procurement/vendor-info/terms-and-conditions/general-provisions-for-goods-and-services/

## AUTHORITY

IFB No. 26-4679 is issued under the provisions of Hawaii Revised Statutes, Chapters 103 and 103D. All prospective bidders are charged with presumptive knowledge of all requirements of the cited legal authorities. Submission of a valid executed bid by any prospective bidder shall constitute an affirmation of such knowledge on the part of such prospective bidder.

## TECHNICAL REPRESENTATIVE OF THE PROCUREMENT OFFICER (TRPO)

A designee from the Commuter and Fleet Services Department will serve as the Technical Representative of the Procurement Officer.is Cate Matsushima, Commuter & Fleet Services Manager, telephone (808) 956-9290.

## SUBMITTAL OF TECHNICAL DATA

With their bids, bidders shall submit, in duplicate, manufacturer's literature or brochures with technical data and illustrations of the vehicle being offered.

- BIDDER'S QUALIFICATIONS

To qualify to bid on the specified vehicles, the bidder must be engaged in a business whose primary and customary interest is to provide the specified vehicles.  The bidder must also have the requisite experience, appropriate forms of insurance, and proper licenses.  The University reserves the right to disqualify any potential bidder if, in its discretion, the University determines that the bidder does not have the requisite experience or expertise to provide the goods and/or services.

## DELIVERY

Prior to delivery, the Contractor shall contact the Technical Representative to coordinate delivery of the vehicles.

The Contractor shall deliver the vehicles within one-hundred and twenty (120) consecutive calendar days from the date designated in the Notice to Proceed.

The physical site at which the vehicles shall be delivered is as follows:

2027 East West Road

Honolulu, HI 96822

## MANUALS AND INSTRUCTIONS

The Contractor shall provide the University with operating and maintenance manuals of the vehicles furnished under this contract except when hardcopy manuals or CD versions are no longer produced by the vehicle manufacturer for distribution. If available, the contractor shall also provide any manufacturer-issued repair manual.

- FACTORY PROVIDED EQUIPMENT

All vehicles furnished under this contract shall include factory-equipped roadside accessories consisting of a spare tire (temporary or full-size, as applicable) and a vehicle jack.

- KEYS

Each vehicle shall be delivered with two (2) sets of original manufacturer keys, each equipped with remote entry functionality.

- CHARGING EQUIPMENT (APPLICABLE TO ELECTRIC VEHICLES)

All electric vehicles furnished under this contract shall include the manufacturer’s standard portable charging cord or charging cable compatible with Level 1 or Level 2 charging (as applicable to the vehicle). Charging equipment shall be new, functional, and included at no additional cost to the University.

- AVAILABILITY OF FUNDS

Offerors are advised that the award of this contract is contingent upon availability of funds.  If funds are not available, the University reserves the right to cancel the IFB or not award individual items.

- DAMAGED/DEFECTIVE VEHICLES

The University reserves the right to reject damaged or defective vehicles, which shall be promptly and satisfactorily replaced by the Contractor at no additional cost of the University

- PAYMENT

Contractor shall be remunerated upon final acceptance of goods and submission of a properly executed original invoice and ONE (1) copy, indicating the contract number to (1960 East-West Road, Room T111, Honolulu, HI 96822), THIRTY (30) calendar days following submission of invoice and acceptance of services/goods.

## MAINTENANCE AND SERVICE REQUIREMENTS

The Contractor shall ensure that all vehicles furnished under this contract are maintained in accordance with the manufacturer’s recommended service schedule and shall remain in safe, operable condition throughout the warranty and service period. All maintenance and repairs shall be performed by a manufacturer-authorized service facility located within the State of Hawaiʻi, using genuine Original Equipment Manufacturer (OEM) or manufacturer-approved parts, in compliance with HAR §3-125-5 (Quality Requirements).

The Contractor shall provide all necessary preventive maintenance, inspections, and repairs at no additional cost to the University during the warranty period, and shall maintain complete service records for each vehicle. If any vehicle is out of service for maintenance or repair for more than fifteen (15) consecutive calendar days, the Contractor shall provide a comparable loaner vehicle at no cost to the University until the vehicle is returned to full operational condition.

All vehicles are subject to inspection and acceptance by the University pursuant to HAR §3-125-14 (Inspection and Acceptance). Failure to comply with these requirements may be grounds for contract termination under HAR §3-125-18 (Termination for Default).

## VEHICLE WARRANTY AND PARTS SUPPORT

In accordance with HAR §3-125-13 (Warranty Clauses) and HAR §3-122-112 (Responsibility of Offerors), the Contractor warrants that all vehicles, equipment, and associated parts furnished under this contract are new, unused, and free from defects in materials, workmanship, and title.

The warranty period shall be not less than SEVEN (7) years or 75,000 miles, whichever occurs first, unless the manufacturer’s standard warranty provides greater protection. The Contractor shall honor and enforce all manufacturer warranties and ensure that warranty coverage extends to the University of Hawaiʻi as the end user.

The Contractor shall maintain, or have access to, an authorized service facility within the State of Hawaiʻi capable of performing warranty repairs, parts replacements, and maintenance. All replacement parts shall be genuine Original Equipment Manufacturer (OEM) components or manufacturer-approved equivalents, and shall remain available for at least seven (7) years following vehicle delivery.

Pursuant to HAR §3-125-13(b), during the warranty period the Contractor shall, at no additional cost to the University, correct any defects by repair, replacement, or adjustment, including all related labor, materials, and transportation. If a vehicle is out of service for warranty repair for more than fifteen (15) consecutive calendar days, the Contractor shall furnish a comparable loaner vehicle at no cost to the University until repairs are completed.

All warranty documentation, including terms, authorized service locations, and claim procedures, shall be furnished to the University upon vehicle delivery. The Contractor shall ensure that warranty claims are processed in a timely manner consistent with HAR §3-125-14 (Inspection and Acceptance) and that all repairs meet applicable safety and performance standards.

## PAYMENT

The Contractor shall be remunerated upon satisfactory delivery and acceptance of the vehicle, and submission of a properly executed original invoice and ONE (1) copy, indicating the contract number, to Auxiliary Services Fiscal Office, 1951 East-West Road, Honolulu, Hawaii 96822.

## CERTIFICATES REQUIRED

Prior to delivery, the Contractor shall obtain the following and deliver to the University together with the vehicle:

  - Hawaii Safety Inspection Certificate (in duplicate) and decal;

  - Motor Vehicle Registration Certificate (completed and signed); and

  - Odometer Certificate.

## STATE LICENSE FOR SELLING MOTOR VEHICLES

a. Pursuant to Chapter 437, Hawaiʻi Revised Statutes (HRS), relating to the regulation and licensing of motor vehicle manufacturers, distributors, dealers, salespersons, auctions, and auctioneers, all prospective bidders engaged in the business of selling or purchasing motor vehicles in the State of Hawaiʻi shall hold a valid and current State of Hawaiʻi Motor Vehicle Industry License issued by the Motor Vehicle Industry Licensing Board.

b. In accordance with HAR §3-122-112 (Responsibility of Offerors), the University of Hawaiʻi reserves the right to verify the bidder’s licensing status as a condition of determining bidder responsibility. Bidders shall provide proof of such licensure by listing their current license number and expiration date in the space provided on the Bid Form or by attaching a copy of the valid license with their bid submission. Failure to comply with this requirement may be grounds for rejection of the bid as non-responsive.

c.  For purposes of this solicitation, “Motor Vehicle” includes any vehicle, automobile, or truck as defined in HRS §§249-1 and 286-2, but excludes tractors, trailers, and amphibious vehicles.

- VERIFICATION AND REVOCATION OF MOTOR VEHICLE LICENSE

a. The University of Hawaiʻi reserves the right to verify the status and validity of any bidder’s required State of Hawaiʻi Motor Vehicle Industry License with the Department of Commerce and Consumer Affairs (DCCA), Professional and Vocational Licensing Division, or the Motor Vehicle Industry Licensing Board.

b. If, at the time of bid opening or contract award, the bidder’s license is expired, suspended, revoked, or otherwise invalid, the University may deem the bidder non-responsible and reject the offer. No award shall be made to a bidder whose license is not valid and in good standing at the time of award.

c. The Contractor shall maintain a valid license for the entire duration of the contract. Failure to maintain an active and valid license shall constitute a material breach of contract and may result in termination for default pursuant to HAR §3-125-18.

d. The University may, at its sole discretion, periodically request written confirmation of license validity during the contract term. The Contractor shall provide such documentation within five (5) business days of the request.