2025 or Newer Trailer Minium Specifications

One 38’, 40k GVWR Trailer, delivered to Division of Forestry and Wildlife 4398D Pua Loke St. Lihue, HI 96766.  Minimum specifications below

GVWR – 40,000lbs, 40,000 lb GVWR Package

Length/Width - 38' x 102"

Axles - 2 - 16,000 lb Heavy Duty, Oil Bath

Dovetail – Full Steel Runner with Cleats, rated to lift and carry 10,000lbs distributed across the length of the dovetail.

Brakes - Electric Over Hydraulic Disc

Suspension - Hutch Heavy Duty Adjustable Suspension

Pull Type - 12" Engineered Gooseneck

Frame - 18" Engineered Beam 2.0

Loading Style – 12’ Hydraulic Dovetail

Floor - 2" Oak Floor

Cross-Members - 3" I-Beam on 16" Centers w/ Formed Gussets

Coupler - 3" ball - 40,000 lb Adjustable Telescoupler

Battery/Charger - Group 27 Series Battery w/110V 5amp Charger

Battery/Charger - Solar Charging System, 7 watt

Winch – Winch Tray

Storage - Diamond Plate Neck Box w/ Gas Shock Assisted Lid & 14" x 14" x 42" Underslung Box

Jack – 2 - 20,000 lb Hydraulic Jack

Steps - Front Retractable & Mid-Turn & Extra Retractable Steps (pair)

Lights - All LED Lights, Bolt-on LED Loading Lights, 4 Pair ¾” Clearance Lights

Lace Rail - 6" x 2" Rectangle Tube Lace Rail

Sway Control- Included

Stake Pockets - 2" x 3/8" Rub Rail w/ Stake Pockets & Pipe Spools

Tire Size - 235/75R17.5 18 Ply, Radial, Dually

Trailer and Wheel Color - Black

Spare Mount - Under-Frame Spare Mount

Spare Tire - Add Matching Spare Tire