STATE OF HAWAII

DEPARTMENT OF LAND AND NATURAL RESOURCES

DIVISION OF FORESTRY AND WILDLIFE

KAUAI, HAWAII

INVITATION FOR BID (IFB)

FOR

FURNISHING AND DELIVERING OF SEMI TRUCK AND TRAILER

ISLAND OF

KAUAI

### ITEM 1 - SEMI TRUCK

SCOPE OF WORK - The work consists of furnishing and delivering one (1) NEW Semi Truck.  Contractor shall present the vehicles / equipment complete, fully operational, and ready to use.

SEMI TRUCK - The Specifications contained herein establish minimum standards (unless stated differently). Vehicle / equipment offered shall include any other standard features not listed but detailed in manufacturer’s brochures and deemed necessary for proper and safe operation of vehicle/equipment.  Vehicle must have current safety inspections at time of delivery.

- Model/Year				Kenworth T880 or similar; latest production.

- Engine/powertrain			Paccar MX13 510HP, 1850 lb-ft of Torque or Cummins

X15 510HP, 2,000 lb-ft or Torque

TruckTech+ Remote Diagnostics

- Exhaust/Aftertreatment		Standard

RH Under DPF/SCR with vertical side cab tailpipe

Small Round DEF Tank – 11 usable gallons

- Transmission				18 Speed Manual

- Electrical				Standard options

Multiplex electrical system with centralized power distribution incorporating plug-in style relays & circuit protection for serviceability

12 Volt lighting system with circuit protection

Circuits numbered and color coded

Key switch start & shutdown

- Starting and Charging		Standard options

160 Amp Alternator

PACCAR 12 volt starter

Three PACCAR Dual purpose batteries

- Frame				Standard options

10 5/8” Steel – 120,000 psi – Heat treated

- Axle and Suspension			Standard options

Front axles – Meritor, 12,500 lb.

Front springs – taperleaf w/shocks, 12,000 lb

Rear axle – PACCAR, 40,000 lbs. Tandem

Rear suspension – Reyco 102, 38,000 lbs. tandem

- Rear Axle ratio			4.56 gear ratio

- Brakes				Standard options

Front – Air Disc

Rear – Air Drum

ABS, ATC, ESP – Bendix

- Wheels/Tires				Standard options

Wheels – steel 22.5”

Bridgestone 295/75R22.5

- Fuel tanks				Standard options

24.5” Round aluminum 100 gallon capacity

- Batter boxes				Standard options

Steel battery box with aluminum step cover

- Bumpers				Standard option

Aluminum channel bumper

- Cab/Hood/Interior			Standard options

Cab – stamped aluminum

Hood – 5 piece Metton Sloped vocational hood with bolt on fenders, MX Hood -  116.7” BBC

Complex reflector halogen headlamps

Aerodynamic breakaway heated mirrors

Adjustable telescoping tilt steering column

Standard vintage vinyl headliner and cab back panel

High back driver seat

High back toolbox plus rider seat

Power door locks

Passenger=side electric window

- Driver assistance technology	Standard options

Stability control

Lane departure warning

- Special equipment and options	Backup alarm

Two line pressure and return wet-kit

Auxillary air and electrical switches, and two way tarp switch

- Color					White

- Training:

The contractor shall provide operator and maintenance training to demonstrate the proper and safe operation of equipment. This training shall consist of a minimum of one (1) hour class and one (1) hour hands-on training, per operator. The contractor shall provide a classroom site. This service shall be provided at no additional cost to the State. Training shall be consistent with OSHA requirements.

### ITEM 2 – 40-TON SLIDING AXLE TRAILER

SCOPE OF WORK - The work consists of furnishing and delivering one (1) NEW Trailer.  Contractor shall present the vehicles / equipment complete, fully operational, and ready to use.

TRAILER - The Specifications contained herein establish minimum standards (unless stated differently). Vehicle / equipment offered shall include any other standard features not listed but detailed in manufacturer’s brochures and deemed necessary for proper and safe operation of vehicle/equipment.  Vehicle must have current safety inspections at time of delivery.

- Model/Year				Trail King TK8OSA or similar; latest production.

- Type:					Hydraulic 40 ton-sliding axle trailer

Perimeter frame gooseneck

- Payload Capacity:			80,000 lbs uniform/50,000 lbs in 10 ft

- Dimensions:				48 ft. (14.6m) overall length max. 102 in. (2.59m) min

Upper Deck length 9 ft. 10 in. (2.77m) max

Gooseneck – shall clear when jack knifed. 16 in. kingpin setting with 80 in swing clearance

Flange material shall be 100,000 psi yield strength steel, min

Web material shall be 80,000 psi yield strength steel, min

Deck cross members shall be 80,000 psi yield strength steel, minimum

Heavy duty swing-out outriggers, to include additional deck supports (to allow for extra width of deck).

- Decking:				2 in. (50.8mm) thick longitudinal hardwood/apitong flooring

- Suspension:				Manufacturer’s standard air ride system.

- Wheels:				Nine (9) each, manufacturer standard.

- Tires:					Nine (9) each, 17.5 16PR radial tires, spare included,

tubeless

- Brakes:				Air type disc brakes on all wheels; shall be equipped with

parking brakes

Anti-lock brake system 4S2M minimum

Automatic slack adjusters

- Axles:				Manurfacturer’s heavy duty sliding axles to meet payload

capacity provided. No travelling of hydraulic lines when sliding axles are in operation.

- Color:				White; high gloss urethane or manufacturer equal.

- Other:

- Twelve (12) lashing rings, six (6) on each side of trailer. Eight (8) transport chains with hook-ends 20 ft. (6,096 mm) lengths and eight (8) ratcheting chain, binders.

- All light assemblies shall be sealed LED type.

- King Pin - shall be located so front end of trailer will not interfere with any components when jack knifing.

- All circuits shall be wired through automatic reset circuit breakers; wiring shall be in conduit and mounted with rubber grommets; wiring gauge size shall be adequate to provide voltage to all lights.

- All lights, including directional tum signals, arrow for right/left tum and third tail light shall conform to PUC, MCB #2. LED type.

- Deck Height - shall be a maximum 37.5 in. (952.5 mm) loaded.

- 20,000 lb. hydraulic winch mounted on front upper deck with winch roller, skid plate and winch guard, shall have two (2) each full function remotes for both winch and trailer functions and dual side hydraulic controls.

- Two (2) lockable tool wells located in gooseneck front deck.

- Shall be equipped with selector valve in order to be run off of the tractor wet line kit.

- Shall have Hub odometer.

- Shall be equipped with mid ship turn signal.

- Shall be outlined with conspicuity material (reflexite or equal) to meet Federal requirements; shall match truck tractor.

- PUC Certification, License, Registration, and Certificates as required in General Specifications.

- Two (2) each Operator's Manual; Two (2) each Trailer Service/Repair Manual; Two (2) each Parts Manual. Book format.

- Contractor shall provide a list of factory-trained and authorized personnel capable of providing installation and service support.

- Contractor shall provide a list identifying those items and systems which require factory-authorized personnel to install and maintain. Contractor shall also stock all necessary parts for the same.

- First Service Kit to include all required filters.

- Complete after factory rustproof with minimum seven (7) year unlimited mileage requirement warranty. In accordance with Federal Specifications 297 A at its latest

- Backup alarm.

- Anti-sail mud flaps.

- Upon 11Notice to Proceed", Contractor shall submit "Application" (To State Motor Vehicle

Safety Office) to meet the requirements of Section 286-202 (12 f Hawaii Revised· Statutes," .

for approval of a major modification. Construction, or reconstruction of a vehicle with a

gross vehicle weight rating (GVWR) of 10,001 lbs. and above.

Copy of approved application shall be submitted a time of delivery.

- Training:

Contractor shall provide operator and maintenance training and refresher training one (1)

year after delivery to demonstrate proper and safe operation of vehicle and equipment. This

training shall consist of a minimum of one (1) hour class and one (1) hour hands on training,

per operator. Contractor shall provide classroom site. This service shall be provided at no

additional cost to the State. Training shall be consistent to OSHA requirements.