# SPECIFICATIONS

REPLACE EXISTING CARPET WITH NEW VCT TILES AT KEKUANAOA BUILDING AND STATE OFFICE TOWER ON OAHU

CSD-26-027-O

SCOPE OF WORK:

The Contractor shall provide all labor, materials, tools, vehicles, and equipment necessary to remove existing carpet and install new Vinyl Composite Tile (VCT) flooring install new rubber floor bases; and touch up painting. The State may task the contractor to perform other repairs and finishing work.

The itemized scope of work is as follows:

- Relocate/Store furniture to replace flooring.

- Remove existing carpet, padding, tack strips, base and all necessary to prep concrete surface to accept VCT flooring.

- Install new 12” square x 1/8” VCT flooring and rubber wall bases. Color and pattern to be determined. Armstrong Flooring Premium Excelon Imperial Texture or approved equal. Provide submittal and sample product data for review prior to order and installation. Polish new tile with floor polish S-480 or equal.

- Touch up paint all walls.

- Properly dispose of all removed building items.

- Reinstall furniture.

- Attachment SK-1 and SK-2 locates the area or work.

Qualifications of the Bidders:

If requested by the State, contractor may be asked to provide documents demonstrating their Contractor or Contractor’s Responsible Managing Employee (RME) Qualifications. Documents, at the time of bidding, include:

- A valid State of Hawaii, “B” General Building Contractor’s License. The Contractor shall possess the license throughout the duration of the contract and or any extensions. The Contractor shall provide a copy of the specified license and contractor license number as applicable.

- The contractor shall have a minimum of two (2) consecutive years of Hawaii “B” General Building Contractor experience. The experience shall include a minimum of two (2) years in the managing contracts like this project in size and scope.

- The State may request a list of similar projects to substantiate the bidder’s/contractor’s experience. The list shall contain a minimum of three (3) different projects and shall include a) name of the company, b) point of contact, c) phone number, d) description of landscaping work performed, and e) size of the project (cost wise).

Additionally, prior to issuance of an NTP, the awardee shall provide a certificate of liability insurance to the levels required in Section 23.0 LIABILITY INSURANCE of the Special Provisions.

Quality of Work:

All services performed will be subject to inspection and correction of any substandard work as requested by the contracting officer. Provide a 1-year written material and labor warranty on all work – Contractor will not be held liable for acts of user vandalism.

Safety:

Contractor shall close off area(s) and post signs indicating the area(s) are closed to pedestrian traffic and walkways. Equipment, apparatus, or equipment on the ground shall be marked off with cones and signs warning pedestrian traffic. Contractor shall provide all safety cones/barriers and signs.

Reporting Damages:

Any damage that are discovered prior to the start of work and shall be reported to the contracting officer.

Any damage to buildings exteriors, structures, vehicles, plants and other items in the area and adjoining properties caused by the Contractor or any of its personnel shall be repaired and/or replaced by the Contractor at their own expense and to the satisfaction of the contracting officer or his designated representative and any injured party.

Scheduling:

Work shall be performed within 30 calendar days from a written notice to proceed. Contractor will coordinate with the Contract Administrator or their designated representative on the work schedule prior to the issuance of the Notice to Proceed. Schedules of work may change or be adjusted at any time, upon mutual agreement by both parties.

Parking:

The Contractor will coordinate, obtain and pay for all required parking permits and passes to utilize parking facilities. See Special Provisions for additional information.

Bid Submittals:

Bid Prices shall include all costs for all labor, equipment, supplies, applicable taxes (including the current Hawaii General Excise Tax) and any other expenses incurred to provide all the specified work, complete and in-place.

Extra Work:

The Contractor shall submit a separate Proposal for all work items not covered by the contract. This work is additional to the Contract and is subject to review and approval by the Contract Administrator. If the Proposal is found acceptable, and without further solicitation of other Bidders, the Contract Administrator shall authorize the work. Payment for extra authorized work will be by Purchase Order and/or Purchasing Card.

Contract Administrator:

Contract Administrator shall be James Kurata, DAGS/CSD, or designee, Teri Wong, Central Services Division Engineer. She may be contacted by e-mail at teri.k.wong@hawaii.gov.