DETAILED SPECIFICATIONS

PATROL VESSELS

HAWAII COMPLIANCE EXPRESS COMPLIANCE:   IMPORTANT - The awarded vendor must have compliant status within the Hawaii Compliance Express system AT THE TIME OF AWARD.  Offers submitted by vendors who do not have compliant status will NOT BE ACCEPTED AND WILL BE DEEMED NON-RESPONSIVE.

COMPLETION AND DELIVERY:  IMPORTANT - The awarded vendor must have the vessels completed and delivered no later than 12/31/2026

MINIMUM SPECIFICATIONS:

Two 2026 or latest production model, twenty-foot (20’) hybrid deep “V” tri-hull fiberglass Sea Blade vessels or equivalent, with State of Hawaii and Federal DOT rated vessel trailers, gasoline powered 90 horsepower outboard marine rated engines, delivered to the State of Hawaii, Department of Land and Natural Resources-Division of Conservation and Resources Enforcement, Honolulu, HI.  The described vessels shall be drop shipped as specified.

Construction:

- Length Overall (LOA) with engines: 23’2”

- Beam: 8’1”

- Fiberglass Center console

- Self‐draining Cockpit Sole

- Storage in center console (36” W x 39” L x 55” H)

- 17.5” gunwales

- Vacuum infused hull, deck, stringers and bulkheads.

- Optimized layup using E-QXCFM 3515, 1708, Soric TF 2mm, and Soric XF 4mm

- Foam-filled and fiberglass-capped below deck cavities

- Hybrid deep-V, Tri-hull design optimized for Hawaiian ocean waters.

- Kevlar beaching pad

- Full foam collar

Propulsion:

- The most current twin 4-stroke, 4-cylinder, 90 horsepower Suzuki marine outboard engines or equivalent

Console:

- 30”x13” electronics mounting surface

- Two cupholders on starboard side of console

- Integrated footrest

- Instrumentation and electrical breaker panel

- Step down tub for storage and/or utility space

- Neptune III or equivalent leaning post

- Two (2) helm boxes with a USB charging station

- Marine rated digital revolutions per minute (RPM), miles per hour (MPH) and fuel gauge package

- Polished stainless steel steering wheel with a stainless nut and stainless power knob to be located slightly off-centered port side of the console

- Fly-by-wire top mount shift and dual throttle controls located on the console, port side of the steering wheel

- Key switch and emergency cut off to be located below the steering wheel

Outfitting:

- Windscreen:

  - 3/8” thick

  - Width of console

  - Extends 16.5” above the top of the console

  - Sides extend 17” from the front of the console

  - Spacers installed between windscreen and console to allow proper drainage

- Zipwake Dynamic Trim Control System or equivalent

- Stainless steel dive ladder which can be secured and stowed on the vessel

- T-top:

  - 2” schedule 40 anodized aluminum supports with standoff to console

  - Canopy 90” length x 66” Width

  - Sunbrella shade (Black, Green, or Blue)

- Nine (9) 8” Stainless Steel Cleats (one for anchor, four on port and starboard sides)

- Sled anchor cleats on both the port and starboard side quarter

- Stainless steel bow roller eye

- Footrest integrated into console

- WeatherDeck Circuit Breaker Panel or equivalent mount at the helm station

- Stainless steel bow rail

- Sahara MK2 1200 GPH or equivalent bilge pump

- Raised foam core in bilge to ensure the bilge pump is located at the lowest point in the hull.

- AMG Batteries or equivalent

- Toe bar on port and starboard sides

- Single layer of SeaDek or equivalent non-skid EVA deck foam

- Neptune III Leaning Post

- Llebroc 47” Billfish Bench Seat or equivalent

- T-top mounted Shakespeare antenna or equivalent

- Engine guard

- A fixed and secure stainless steel Sampsons post located abaft of the vessel for aft tows

- Heavy duty rub rail

- Bow tow eye – secondary tow eye, in addition to standard towing eye for use of two (2) tow bridles.

Electronics and Navigation:

- VHF Radio-Horizon/ICOM or equivalent

- Garmin VHF115 with antenna or equivalent

- Garmin GPSMAP 943 with transducer or equivalent

Lights:

- Hella Marine Naviled Deck Mount or equivalent port & starboard pair – 2 nautical mile - Clear Lens/White Housing Navigation Lights

- Fore and aft flood light package, and courtesy lighting

Law Enforcement Vessel Warning Lights, Siren & Public Address System:

- Contractor shall provide and install the following specified equipment for a fully functional system upon delivery. Placement of equipment prior to installation shall be with full consultation, and approval of the officer in charge. All necessary marine grade stainless fasteners, fabricated brackets, marine grade wiring, fuse block, and switching, plus master cut off switch shall be the responsibility of the installer.

- Warning Lights: Install on T-Top 2 pairs (total of 4 each) Whelen WIONB blue warning lights, one facing forward, one facing the aft, and one for both starboard and portside.  The installer shall fabricate a plate to mount the work lights located above the T-top.  Placement to be consulted and finalized by the officer in charge.

- Work Lights: Install two (2 each) Whelen NP6BW Pioneer Nano work lights or equivalent. The installer shall fabricate a plate to mount the work lights located above the T-top.  Placement to be consulted and finalized by the officer in charge.

- Siren / P.A. / Speaker system: Install a Whelen model WPA112 waterproof remote siren amplifier or equivalent, with a Whelen model WPA3BM control head with PA or equivalent, and Whelen model SA315P 100-watt siren speaker or equivalent.  Installer shall fabricate stainless steel mounting bracket for siren speaker above the T-top.

- Final placement shall be consulted and determined by the officer in charge.

- Each lighting system shall have assigned utility switches mounted to the console and breaker switches.

Fuel:

- 98-gallon aluminum belly fuel tank located at the center of gravity of the vessel

- Fuel fill located on the starboard gunwale

- Reed switch fuel sender

- Two Racor or equivalent fuel filters located in the aft bilge with shutoff valves to prevent fuel siphoning

Trailer:

- Aluminum EZLoader 5200# Trailer with Decromet coated brakes or equivalent

- Front bump stop

- Bow catch

- The trailer shall have the applicable tires, tire ply and Department of Transportation compliant