STATE OF HAWAII

DEPARTMENT OF LAND AND NATURAL RESOURCES

DIVISION OF FORESTRY AND WILDLIFE

KAUAI, HAWAII

INVITATION FOR BID (IFB)

FOR

REVISED

4-20-26

FURNISHING AND DELIVERING OF SEMI TRUCK AND TRAILER

ISLAND OF

KAUAI

SCOPE OF WORK - The work consists of furnishing and delivering one (1) NEW Semi Truck.  Contractor shall present the vehicles / equipment complete, fully operational, and ready to use.

### ITEM 1 - SEMI TRUCK

ONE (1) ONLY 59,200 LB. GVWR TRUCK TRACTOR

Model:	New, 2026 or current production model Peterbilt 567, or equal

Type:	6x4 Set-Back Front Axle with a top speed of 65 mph

GVWR:	59,220 lb.

Dimensions:	Wheelbase: 195" Minimum Cab to axle: 127"

Frame Rail Overhang: Maximum 54"

Frame:	Frame rails to be heat treated zinc phosphate coated alloy steel (120,000 psi yield), 10.75" x 3.50" x 0.375" x 304" overall length including frame rails with tapered rear 3 section 15" wide, aluminum deck plates

Frame

Reinforcing:	Full inner "C" channel, heat treated alloy steel (120,000 psi yield)

9.50" x 3.11" x 304”

Front Bumper:	Full width, polished aluminum/stainless steel, deep tapered Horsepower:	Minimum 500 hp @ rated governed speed

Torque:	1850 lbs/ft torque @ rated rpm

Engine:	Diesel fueled, 2017 Emission Certified Cummins ISX15 525, or equal, 50 state

- Heavy duty starter motor

- Dash mounted air cleaner restriction gauge

- Key operated engine shut down

- Electronic governor

- Electronic throttle hand control, instrument panel mounted

- Magnetic oil drain plug

- Spin on type oil filters

- Timer idle shutdown

- Engine water filter

- Engine fuel filter

- Engine compression brake, 3 position or equal with selector switch

Engine Warranty:	Engine manufacturer’s new engine warranty shall apply

Transmission:	Eaton (18) Manual transmission with appropriate Engine Torque

matched clutch assembly.

Reverse alarm:	Electronic beeper type, tied in with reverse lights. Alarm shall be

between 85-90 dba @ 20 ft. from rear of truck

Front Axle:	I-beam type, 14,600 lb capacity minimum Front

Suspension:	Tapered leaf front springs, 14,600 lb capacity with shock absorbers

Rear Axle:	Tandem, Spicer D46-170, Meritor RT46-160 or equal, single

reduction, 46,000-lb capacity 4.56 gear ratio, to include magnetic drain plugs, power divider lock air operated cab mounted with indicator light, oil pump, 0.50 wall thickness

Rear

Suspension:	Tandem, Hendrickson RT-463 or equal, walking beam type, 54" axle

spacing, 46,000-lb capacity with leaf springs and bronze center bushings, including beam end adapters-bar pin type

Cab:	121” BBC Conventional all aluminum, riveted or huck fastened, no steel reinforcements; rear window shall include three (3) separate viewing windows for maximum viewing and operator safety (no exceptions). 83" minimum width between the LH front cab pillar to the RH front cab pillar; 22" minimum width between driver and passenger seats

- Front end tilting assisted fiberglass hood

- Chrome plated or stainless steel surround grill

- Chrome plated or stainless steel headlight bezels

- Storage pocket mounted cab rear panel or both doors

- Cab interior grab handles, 1 each side

- Heater with defroster and integral air conditioning

- Two (2) each grab handle towel bar type

- Two (2) each courtesy lights, driver & passenger door mounted

- Tinted glass all windows

- Floor mats vinyl rubber

- Two (2) sun visors

- Five (5) clearance/marker lights, roof mounted

- Two (2) each moto-type mirrors w/ integral mirrors, to accommodate trailers up to 102" wide.

- Adjustable steering column. Includes audio volume, seek, mute and mode button on LH pod with cruise control on/off/cancel, set/resume and accelerate/coast on the right pod

Seats:	Driver: Peterbilt HP or equal, air suspension, high back, all vinyl, w/right &

left side arm rests

Passenger: 		Peterbilt ST or equal, high back, all vinyl, non-suspension type

Additional Equipment:

- Door mounted turn signals

- Two (2) strobes cab roof mounted on RH & LH stainless steel brackets with switch & warning light in cab

- 48" air slide 5th wheel, Holland FW70 or equal, 51" to top of wheel on 2" x 3" angle iron or tubing. Rearmost 5th wheel setting shall be set at 6”in. rearward from center of trunnion. LH release. 5th wheel lock for end dump trailer.

- Half stainless steel tandem fenders over rear wheels

- Rear mud flaps on removable spring type hangers

- Coupler mounted on 5/8" plate w/ "A" brace Holland CP-380-A or equal

- Two (2) "D" rings, 7 way socket and trailer air supply with diversion valve at rear

- Install two (2) each frame mounted access steps, one each side or mounted on fuel and hydraulic tank

- Two (2) each 30" grab handles one (1) each side back of cab

- 2 rear flush mounted spot lights behind cab

11) 	Auxillary air and electrical switches, and two way tarp switch

WET KIT:

PTO:	118% - 125% of engine speed. PTO Safety

Interlock:	Chassis to be equipped with safety interlock system which will

allow a maximum of 5 MPH chassis speed when PTO is engaged to prevent potential hydraulic system damage

Pump:	Direct mount P51 series with 2 ½” gear

Reservoir:	50 gallon minimum, aluminum tank, mounted to frame, left hand side, back of cab. Includes grip type steps for access to deck plate.

Suction Line:	2” inside diameter minimum, 5’ length maximum, spiral wire type to

prevent hose from collapsing under suction with shut off valve to be installed between tank and hose.

Pressure Line:	1” inside diameter minimum with diverter valve directing flow to Line

#1 with power beyond valve set at 3000 psi with 1” male quick coupler on valve for end dump and power beyond with 1” male quick coupler for walking floor and Line #2 with pressure relief valve set at 2000 psi with 1 ¼” male quick coupler on hose for push out. Aeroquip 5100 series wing style quick disconnects or comparable with protective caps.

Return Line:	1” inside diameter minimum from trailer to filter. 1¼” inside diameter

minimum from filter to tank.

Filter:	10-30 micron in return line. Sufficient size to properly filter oil when pumping 40-45 gpm. 1¼” male quick disconnect wing style installed on filter head.

Relief valve shall be set at no less than 2800 psi and no more than 3000 psi.

Brake System:	ABS 6S6M system with 6 wheel anti-lock, ATC controls and anchor

lock or maxi spring loaded safety brake system. Front and rear disc type w/ severe service rotors & dust shields

  - Air brake system for truck tractor application including: turn signal flasher trailer connections 15'(coiled nylon) & 15' lighting cable with 7-way connector

  - Dual air pressure gauges with low air warning buzzer & lights

  - Brake pressure indicator, low air pressure warning light and audible alarm

  - Self-returning hand control valve for trailer brakes

  - Color coded nylon air lines with premium chassis lines

  - Glad hands

  - Two (2) hose tenders to secure air lines & hydraulic lines when not in use

  - Combination parking brake valve for tractor & trailer

  - Manual air tank drain valves cable operated

  - Aluminum air tanks

  - Tractor protection valve

Air Compressor:	Bendix TU-FLO 750 or equal, 18.7 CFM or Bendix BA921, applicable CFM for engine to be offered

Air Dryer:	Bendix AD-ISEP or equal with heater

Steering Gear:	Ross TAS-85 or equal, power with 3-spoke steering wheel, molded,

black, 18" diameter

Tires:	Front-11R22.5 Unisteel G159 (Bridgestone R268, Goodyear or equal) 501 rev/mile, load range H, 16 ply

Rear-11R22.5 Unisteel G159 (Bridgestone L320, Goodyear or equal) 490 rev/mile, load range H, 16 ply

Wheels:	Front & rear 22.5 Aluminum Wheel, 10 stud 8.25, hub piloted.

Radiator:	Cross flow, series system, 1440 sq. in area, increased cooling, with tank and sight glass. Radiator hoses premium, rubber

Fan Drive:	Two (2) speed type for frequent stops and starts

Air Cleaner:	Under hood heavy duty with restriction gauge in cab and valuator Drive Shafts:	Inter axle SPL170XL series, main drive shaft SPL250 Lite HT series Exhaust System:	Single RH Side of Cab steel mufflers, chrome tail pipes, vertical,

includes s/s muffler guards and grab handles. Curved chrome stack, chrome finish

Electrical System:

- Standard 12 volt system

- Alternator 160 amp capacity

- Three (3) AGM or equal 12 volt maintenance free batteries, 2400 cca total

- Manual or automatic reset-breakers to replace all fuses except instrument cluster feed

- Headlights Shall be Projection Style Module Headlights

- Two (2) each chrome air horns

- Two (2) work lights, flush mounted back of cab with switch

- Radio, AM/FM/Bluetooth, Standard speakers

- Electronic engine oil pressure gauge

- Two (2) speed windshield wiper, with intermittent feature. Electric single motor, cowl mounted

- DPF Mounted Righthand side under cab

- Battery	box	frame	mounted	LH	under	cab,	aluminum construction, aluminum cover w/ grip strut steps

- Electronic water temperature gauge

- Single electric horn

- Parking light integral w/front turn & rear tail lights

- Head light dimmer switch

- Self-canceling turn signal

- Gauges: voltmeter, air application, trans temp, axles temp w/ protective covers for sensors, engine temp, coolant temp

- Color coded or continuously numbered chassis wiring

- Two (2) reading lights in cab, one above each door with individual switches

Fuel Tank:	Top draw, non-polished aluminum, 26" diameter, 80 gal. minimum,

mounted right side back of cab with locking cap

DEF Tank:	Mounted on RH frame rail, behind fuel tank Fuel/Water

Separator:	Paccar Extended Service or equal with sight bowl

Paint:	Cab painted white.  Frame and under carriage shall be painted black.

Keys:	Three (3) sets of keys shall be provided

Manuals:	Service - 1 each Parts - 1 each Operators - 3 each

Note: Manuals to cover engine, transmission, axles, brakes, power steering system, power take-off, hydraulic wet kit and hydraulic system.

Safety

Requirements:	Truck shall meet all applicable Federal, State of Hawaii and County

of Kauai safety requirements in effect on the date of delivery.

Exhaust Emission

Control:	Shall meet all applicable Federal and State of Hawaii requirements in effect on the date of delivery

Noise Emission

Control:	Shall meet all applicable Federal and State of Hawaii requirements in effect on the date of delivery

Cab and Chassis

Contractor shall also submit with their bid a written statement guaranteeing that all high wear item parts for the cab & chassis are currently in stock at their physical place of business in Hawaii. (NO EXCEPTIONS).

Warranty:	Manufacturer’s standard new equipment warranty shall be applied

Delivery:	Delivery shall be within 300 days of the official notice to proceed.

### ITEM 2 – 40-TON SLIDING AXLE TRAILER

SCOPE OF WORK - The work consists of furnishing and delivering one (1) NEW Trailer.  Contractor shall present the vehicles / equipment complete, fully operational, and ready to use.

TRAILER - The Specifications contained herein establish minimum standards (unless stated differently). Vehicle / equipment offered shall include any other standard features not listed but detailed in manufacturer’s brochures and deemed necessary for proper and safe operation of vehicle/equipment.  Vehicle must have current safety inspections at time of delivery.

- Model/Year				Trail King TK8OSA or similar; latest production.

- Type:					Hydraulic 40 ton-sliding axle trailer

Perimeter frame gooseneck

- Payload Capacity:			80,000 lbs uniform/50,000 lbs in 10 ft

- Dimensions:				48 ft. (14.6m) overall length max. 102 in. (2.59m) min

Upper Deck length 9 ft. 10 in. (2.77m) max

Gooseneck – shall clear when jack knifed. 16 in. kingpin setting with 80 in swing clearance

Flange material shall be 100,000 psi yield strength steel, min

Web material shall be 80,000 psi yield strength steel, min

Deck cross members shall be 80,000 psi yield strength steel, minimum

Heavy duty swing-out outriggers, to include additional deck supports (to allow for extra width of deck).

- Decking:				2 in. (50.8mm) thick longitudinal hardwood/apitong flooring

- Suspension:				Manufacturer’s standard air ride system.

- Wheels:				Nine (9) each, manufacturer standard.

- Tires:					Nine (9) each, 17.5 16PR radial tires, spare included,

tubeless

- Brakes:				Air type disc brakes on all wheels; shall be equipped with

parking brakes

Anti-lock brake system 4S2M minimum

Automatic slack adjusters

- Axles:				Manurfacturer’s heavy duty sliding axles to meet payload

capacity provided. No travelling of hydraulic lines when sliding axles are in operation.

- Color:				White; high gloss urethane or manufacturer equal.

- Other:

- Twelve (12) lashing rings, six (6) on each side of trailer. Eight (8) transport chains with hook-ends 20 ft. (6,096 mm) lengths and eight (8) ratcheting chain, binders.

- All light assemblies shall be sealed LED type.

- King Pin - shall be located so front end of trailer will not interfere with any components when jack knifing.

- All circuits shall be wired through automatic reset circuit breakers; wiring shall be in conduit and mounted with rubber grommets; wiring gauge size shall be adequate to provide voltage to all lights.

- All lights, including directional tum signals, arrow for right/left tum and third tail light shall conform to PUC, MCB #2. LED type.

- Deck Height - shall be a maximum 37.5 in. (952.5 mm) loaded.

- 20,000 lb. hydraulic winch mounted on front upper deck with winch roller, skid plate and winch guard, shall have two (2) each full function remotes for both winch and trailer functions and dual side hydraulic controls.

- Two (2) lockable tool wells located in gooseneck front deck.

- Shall be equipped with selector valve in order to be run off of the tractor wet line kit.

- Shall have Hub odometer.

- Shall be equipped with mid ship turn signal.

- Shall be outlined with conspicuity material (reflexite or equal) to meet Federal requirements; shall match truck tractor.

- PUC Certification, License, Registration, and Certificates as required in General Specifications.

- Two (2) each Operator's Manual; Two (2) each Trailer Service/Repair Manual; Two (2) each Parts Manual. Book format.

- Contractor shall provide a list of factory-trained and authorized personnel capable of providing installation and service support.

- Contractor shall provide a list identifying those items and systems which require factory-authorized personnel to install and maintain. Contractor shall also stock all necessary parts for the same.

- First Service Kit to include all required filters.

- Complete after factory rustproof with minimum seven (7) year unlimited mileage requirement warranty. In accordance with Federal Specifications 297 A at its latest

- Backup alarm.

- Anti-sail mud flaps.

- Upon 11Notice to Proceed", Contractor shall submit "Application" (To State Motor Vehicle

Safety Office) to meet the requirements of Section 286-202 (12 f Hawaii Revised· Statutes," .

for approval of a major modification. Construction, or reconstruction of a vehicle with a

gross vehicle weight rating (GVWR) of 10,001 lbs. and above.

Copy of approved application shall be submitted a time of delivery.

- Training:

Contractor shall provide operator and maintenance training and refresher training one (1)

year after delivery to demonstrate proper and safe operation of vehicle and equipment. This

training shall consist of a minimum of one (1) hour class and one (1) hour hands on training,

per operator. Contractor shall provide classroom site. This service shall be provided at no

additional cost to the State. Training shall be consistent to OSHA requirements.