UV Photo-Oxidation Unit Specification Requirements for HiePRO purchase

The Natural Energy Laboratory of Hawaii Authority (NELHA), an attached agency of the Department of Business, Economic Development and Tourism is soliciting bids for the purchase laboratory equipment of a photo-oxidation unit for the Water Quality Laboratory.  A complete unit for the liberation of inorganic nitrogen and phosphate from organically bound compounds for the total nutrient analysis of water samples. The equipment consists of a cylindrical lamp housing containing 12 – 100mL quartz sample tubes that surround a 1200w medium pressure photochemical lamp cooled by an integral fan.  The power supply includes a timer for programming exposure.

The complete equipment setup must contain the following items:

- Lajo lamp housing

- Cased power supply, 220v, 50/60Hz

- UV immersion lamp, 1200w, 19.375in

- Quartz sample tubes, 100mL, 35x2.5cm, 19/22 joint (quantity = 12)

- Pyrex stoppers, 19/22 (quantity = 12)

A product that would meet these requirements is manufactured from AceGlass, catalog # 7900-31; similar products will be considered.