Kalaeloa Backfill Pool and Capping of Utilities

SCOPE OF WORK

1.0 Scope of Work

This project is located at Kalaeloa next to Building 1874.

91-1227 Enterprise Avenue,

Kapolei, HI, 96707

The purpose of this project is to fill in an unused swimming pool to grade, cap the utility lines that used to be used solely for the pool, and to demolish and properly dispose of the nearby abandoned utility infrastructure, components, and miscellaneous objects. Detailed information about the pool, what objects to demolish, and what utilities need to be terminated/capped can all be found in the attachment titled Kalaeloa Backfill Pool and Capping of Utilities Drawings.

The Contractor shall provide all labor, tools, equipment, and materials to perform all work related to the filling of the pool, capping of utility lines, and the demolition and disposal of the nearby utility infrastructure and miscellaneous objects. At the end of the project, there must be no tripping hazards or walking obstructions.

The Contractor shall also be responsible for properly restoring the grass that has been damaged during the project.

  - Filling of the Swimming Pool

The Contractor is to first drain the swimming pool and then fill with a layer of base course before applying a top layer of Portland Cement Concrete (PCC) 1 foot in height. It is estimated that the volume of the swimming pool is 8,500 ft³ (1,150 ft³ of PCC and 7,350 ft³ of base course) after omitting the stairs, steel railing, skimmer holes, drains, and the curvature of the walls. The Contractor shall also fill the skimmer holes and drains with concrete. Before concrete is poured, contractor must compact and test the base course to verify that it has been properly compacted to 95% dry density.

1.2 Demolition

The Contractor is to demolish and dispose of the abandoned utility infrastructure for the swimming pool which consists of numerous objects such as tanks, pumps, pipes, etc. The Contractor is to also dispose of nearby miscellaneous objects that have been abandoned and scattered throughout the area. More details on the relevant objects to demolish and or dispose of can be found in the drawings.

1.3 Capping of Utilities

The Contractor is to cap the abandoned utility lines (2 water, 2 electrical) that used to be used solely for the pool. Details on where to cap can be found in the drawings.

  - Submittals

The following submittals are required with the contractor’s offer:

- Means & methods

- Concrete

- Base Course

- Fittings

2.0 Drawings

Please review the Drawings provided in the attachments. The calculations for the estimated volume of the swimming pool, objects to demolish, and where to cap the utilities can all be in these drawings.

3.0 Special Conditions

All work shall comply with all applicable federal, state, and County of Hawaii laws, rules, codes, ordinances, and regulations.

In the case of conflict between this document and those attached, this document shall prevail.

The Contractor shall schedule their work to ensure that the area is secure at the end of each work-day.

Davis-Bacon Wages must be used by contractor and subcontractor(s).

Working hours for this project are between 7 a.m. and 3:30 p.m. Requests to work outside of these hours shall be submitted in writing no less than 1 week in advance. There is no guarantee that such requests will be approved.

All materials shall be disposed of in accordance with all local, state, and federal laws.

Price submitted on the HIePRO Solicitation is to be the lump sum for the job and should include everything required to complete the entire job including mobilization, etc.

4.0 Bidder Requirements and Price Submission

Contractor agrees to repair or replace the concrete if it were to fail within a specified warranty period.

- Failures include, but are not limited to, the following:

  - Failure to comply with performance requirements.

  - Deterioration of the concrete beyond normal weathering.

- Warranty Period: 1-year Worksmanship Warranty

5.0 Bidder Requirements and Price Submission

Any bid/offer submitted must include:

- Construction Offer Form WITH Bid Bond

- Submittals with specifications

- Hawaii Compliance Express Certificate (HCE)

6.0 Other

- Purchase is subject to availability of funds.