Scope of Work for Tax Map Key (TMK): (1) 2-4-024:033, (1) 2-4-025:029, (1) 2-4-029:037, (1) 2-4-029:038, (1) 2-4-029:066, and (1) 2-4-029:067

The table below contains the State’s current estimate of the schedule and significant dates.  All times are Hawaii Standard Time (HST).  If a component of this schedule, such as "Bid quotes Due Date and Time" is delayed, the rest of the schedule may likely be shifted by the same number of days. Any change to the IFB Schedule and Significant Dates prior to the bid quote due date shall be issued by addendum.

| Event | Date |
| --- | --- |
| Solicitation Release: | 5/15/2026 |
| Question Submittal Deadline: | 5/21/2026, 4:00 P.M. |
| Answers to Questions: | 5/22/2026, 4:00 P.M. |
| Bid quote Due Date and Time: | 5/26/2026, 2:00 P.M. |
| Evaluations | 5/26 /2026– 5/28/2026 |
| Estimated Date for Discussions, if necessary | TBD |
| Estimated Due Date for BAFO, if necessary | TBD |
| Anticipated Award Date: | 5/28/2026 2:00 P.M. |

*Supporting documentation must be either uploaded to HIePRO or submitted to darlene.bryant@hawaii.gov.

A.  SCOPE OF WORK (SOW)

The Kona Low weather events of 2026 brought heavy rains that caused damage to infrastructure and property. To decrease the risk to public health and safety, proposed work in the designated stream and drainage areas is to remove debris, trash, sediment/rock build-up, and unwanted vegetation to keep waterways open to accommodate drainage requirements.

Item 1:  Sediment removal to promote positive waterflow will be accomplished by mechanical means using a combination of three methods – inland, in-water with a platform, and in-water with drive-in equipment. A crane with a clamshell bucket and/or an excavator with a long-reach arm will be stationed along the bank and adjacent area of the waterway to perform the work. Heavy equipment will not be allowed on existing stream levees. For areas where the crane and long-reach arm excavator are unable to access, sediment can be dredged in-water with a platform or equipment can be placed into the waterway. To install the platform, the area in the stream where the platform will enter the stream will be initially dredged from land. A crane or excavator on the platform will dredge the remainder of the sediment unreachable from inland using a clamshell bucket. If the equipment has high enough axles such that the stream is not exposed to the underside of the excavator, then the excavator can be driven into the waterway to dredge areas inaccessible from the inland method. All work will be done to the satisfaction of the ODLO. All sediment, vegetation and debris shall be disposed of off-site at an authorized facility.

State of Hawaii registered herbicides for use in streams shall be applied to the freshly cut surfaces if there is vegetation growing in the waterway. All herbicide applications shall be conducted in compliance with the label directions and comply with all Federal and State laws and regulations. The applicator shall be solely responsible for the efficacy of the application, control, and any off-target damage caused by any herbicide application. Contractors will fill out and submit to ODLO the Herbicide Form if herbicide is used.

Item 2. Semi-annual maintenance to begin at six (6) month intervals after work is completed satisfactorily. Maintenance will be conducted in the designated area of the stream bed with the same scope of work as Item 1.

Item 3. ODLO reserves the right to activate the Vendor for two (2) emergency or unscheduled vegetation clearing outside the standard 6-month interval. The Vendor must mobilize crew and equipment to the designated area within forty-eight (48) hours of receiving written or electronic notice from the ODLO. Emergency work will be billed at the bid quote fee.  All emergency work must adhere to the same scope of work as Item 1.

B.  GENERAL NOTES:

- All work shall be done between 8:00 a.m. to 4:30 p.m., Monday through Friday (excluding State holidays) except with the prior approval of ODLO.

- The Contractor shall be responsible for verifying the location of all utilities and structures. Any damage, injury, and/or loss to any property resulting from the performance of the work shall be the sole responsibility of the Contractor, and shall be remedied promptly by the Contractor at his own expense.

- The Contractor will be provided contact information for the sites to coordinate physical access to the site, if needed.

- The Contractor shall provide safety signs, barricade, and other devices necessary for the safety and convenience of the general public.

- The Contractor shall observe all applicable Federal, State, and Municipal acts, statutes, ordinances, and rules of the performance of this job.

- The bid award shall be based on the availability of funds.

- Submission of bid shall be the evidence that the Contractor understands the scope of the job described herein.

- If erosion and sediment control measures are needed, they will be in place before earth-moving and dredging activities begin.  Functionality of erosion and sediment control devices will be maintained throughout the construction period.

C.  MAP OF AREA FOR WATERWAY WORK:

The work area is the portion of the stream highlighted in the map.

See map on following page.