PROJECT: HDOT Facility Grouted Rubble Paving Installation
LOCATION: State of Hawaii DOT Highways Division, Materials Testing & Research Branch 2530 Likelike Highway, Honolulu, HI 96819
CLOSING DATE FOR SUBMISSIONS: 06/17/2026

1. PROJECT OVERVIEW

The State of Hawaii / Owner is seeking competitive lump-sum bids from qualified, licensed commercial masonry or civil contractors (License Classification A or C-31 required in the State of Hawaii) to install a grouted rubble paving system on-site at the HDOT Materials Testing & Research Branch.

2. SPECIFIC SITE & OPERATIONAL RESTRAINTS

- Restricted Working Hours: All loud construction, heavy equipment operation, and material deliveries are strictly restricted to off-peak hours: 8:30 AM to 3:00 PM, Monday through Friday.

- Facility Access: The contractor must coordinate crew badging, vehicle access, and staging areas to ensure zero disruption to active HDOT testing bays and facility traffic.

- Traffic Control: The contractor is responsible for providing all necessary cones, signs, or barriers to isolate the work zone safely.

3. SCOPE OF WORK & TECH SPECS

The contractor shall provide all labor, materials, equipment, hauling, and mobilization to complete the following in strict accordance with HDOT Standard Specifications Section 612:

- Excavation & Subgrade: Excavate a 75’ L x 15’ W x 2’ D footprint (83.33 CY). Dispose of all spoils off-site legally. Compact subgrade to 95% AASHTO T-180 and lay Class 1 geotextile filter fabric.

- Material Standards: Supply clean, sound local volcanic basalt stone (minimum 5" thickness). Grout must be a standard 1:3 Portland cement-to-sand mix.

- Testing Allowance: The contractor must cooperate with on-site HDOT technicians who will pull grout test cylinders for compression testing.

- Weep Holes & Finish: Install 3" PVC weep holes spaced 8' on center, staggered. Hand-tool all surface joints to a 1/4-inch recess. Clean all excess grout off rock faces within 24 hours to ensure a clean, natural stone texture. Final surface variance must not exceed 3/8" under a 10-foot straightedge.

4. PROJECT DIMENSIONS

- Total Footprint: 75 Feet x 15 Feet (1,125 Square Feet)

- Thickness: 2 Feet Monolithic

- Total Volume: 83.33 Cubic Yards

5. BID BREAKDOWN REQUIREMENT

Contractor quotes must be submitted as a lump sum broken down by these line items:

- Site Mobilization, Facility Badging, and Safety Setup: $___________

- Excavation, Hauling, and Base Prep (including Geotextile): ___________

- Pre-Approved Materials (Basalt Stone, 1:3 Grout Mix, PVC Weep Holes): ___________

- Craft Labor, Masonry Setup, Tooling, and Curing Management: ___________

- Hawaii GET (4.5%): ___________
TOTAL PROJECT LUMP SUM BID: $___________

- CONTACT INFORMATION & SITE VISITS

To schedule an on-site inspection or to submit clarifications, contact:

Rennie Serrano

HDOT Highways Division

Materials Testing and Research Branch

2530 Likelike Highway, Honolulu, Hawaii 96819-2928

☏ (808) 832-3405 ext. 103

✉ rennie.b.serrano@hawaii.gov

Structural Notes & Dimensions

- Finished Surface Elevation: Must align flush with adjacent grade or ditch banks to prevent undermining.

- Stone Specification: Clean, sound Hawaii volcanic basalt. No stone shall be less than 5 inches in any dimension.

- Grout Matrix: Monolithic 1:3 Portland cement/sand mix. Voids must be filled 100% with no air pockets.

- Subgrade: Compacted to 95% AASHTO T-180. geotextile filter fabric (Class 1) to be laid beneath the stones if site suffers from severe soil migration.

- Drainage: 3-inch PVC weep holes placed every 8 feet on center, staggered, to relieve hydrostatic pressure behind/underneath the slab.

◄─────────────────── 75'-0" TOTAL LENGTH ───────────────────►

┌───────────────────────────────────────────────────────────┐ ▲

│ 	○ Weep Hole       	   ○          	         ○        		   │ │

─── PLAN      	│ 	                           					                                │ │ 15'-0"

VIEW        	│                  						                                │ │ WIDTH

│      		         ○           	  	         ○   	   		            ○   │ │

└───────────────────────────────────────────────────────────┘ ▼

◄──────────────────────── 8'-0" o.c. ───────────────────────────► (Staggered Weep Hole Spacing)

15'-0" PAVING WIDTH

◄─────────────────────────────────────────────────────────────────►

┌───────────────────────────────────────────────────────────────────┐ ▲ 2'-0"

─── CROSS 	│ ▓▓▒▒▓▓ [RECESSED 1/4" GROUT JOINTS - TEXTURED FINISH] ▒▒	 	     │ │ TOTAL

SECTION       	│ ▒▒▓▓▒▒▓▓▒▒   2'-0" MONOLITHIC LAVA ROCK & GROUT MATRIX   ▓ 	      │ │ THICKNESS

└───────────────────────────────────────────────────────────────────┘ ▼

========================================================================= ◄─ FILTER FABRIC

░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░ ◄─ COMPACTED SUBGRADE