Fusion Night Vision Binocular System – Minimum Specifications

One (1) new Rix RENV-B 1800-2000 FOM or equivalent, delivered to Division of Forestry and Wildlife,3060 Eiwa Street, Rm. 306 Lihue, HI 96766.

1. System Type

- Helmet-mountable binocular night vision system.

- Integrated thermal imaging and image intensification fusion capability.

- Real-time thermal overlay within the night vision image.

- Articulating dual-pod design with automatic pod shutoff when stowed.

2. Image Intensifier Requirements

- Quantity: Two (2) image intensifier tubes.

- Technology: Gen 2+ or better.

- Phosphor: White phosphor.

- Minimum Figure of Merit (FOM): 1800.

- Maximum FOM accepted: 2000 or greater.

- Minimum resolution: 64 lp/mm.

- Minimum Signal-to-Noise Ratio (SNR): 28.

- Automatic brightness control.

- Bright-source protection.

- Manual gain control.

3. Thermal Imaging Requirements

- Integrated thermal sensor.

- Minimum thermal resolution: 640 × 512.

- Pixel pitch: 12 µm maximum.

- Thermal sensitivity (NETD): ≤30 mK.

- Frame rate: minimum 50 Hz.

- Multiple thermal fusion display modes.

- White-hot anded or orange-hot thermal display options.

- Thermal target highlighting and outline modes.

4. Optical Requirements

- Optical magnification: 1×.

- Minimum field of view: 40 degrees.

- Objective lens focal length: approximately 25 mm.

- Focus range: approximately 0.25 m to infinity.

- Adjustable diopter.

5. Display Requirements

- OLED display.

- Minimum display resolution: 800 × 600.

- Full-color display capability.

- Fusion mode and enhanced display modes.

6. Navigation and Situational Awareness

- Integrated digital compass.

- GPS/navigation capability.

- Azimuth heading display.

- On-screen information display (HUD).

7. Power Requirements

- External battery pack.

- Powered by two (2) rechargeable 18650 batteries or equivalent.

- Minimum operating time:

  - 60 hours in image-intensifier-only mode.

  - 8 hours in fusion mode.

8. Environmental Requirements

- Water resistance rating: IP67 or better.

- Operating temperature:

  - −40°F to +122°F (−40°C to +50°C) or better.

9. Physical Requirements

- Helmet mount compatible.

- Dovetail mounting interface compatible with standard Wilcox-style mounts.

- Maximum system weight:

  - Main unit: 1.3 lb (590 g).

  - Battery pack: 0.25 lb (115 g).

10. Included Accessories

- External battery pack.

- Battery charger.

- Wireless remote control.

- Protective lens covers.

- Carrying/storage case.

- User manual.

11. Warranty

- Minimum manufacturer warranty: 5 years.