SECTION 32 31 13

CHAIN LINK FENCES AND GATES

# GENERAL

## Section includes

##### Fence framework, chain-link fabric, and accessories; excavation for post bases; concrete foundation for posts.

##### Related Sections:

###### Section 03 30 00 – Cast-In-Place Concrete:  Concrete anchorage for posts.

###### Section 31 00 00 – Earthwork: Excavation for post bases.

## REFERENCES

##### ASTM A392-11a(2017) – Standard Specification for Zinc-Coated Steel Chain-Link Fence Fabric.

##### ASTM F567-14a – Standard Practice for Installation of Chain-Link Fence.

##### ASTM F900-11(2017) – Standard Specification for Industrial and Commercial Steel Swing Gates.

##### ASTM F934-96(2017) – Standard Specification for Standard Colors for Polymer-Coated Chain Link Fence Materials.

##### ASTM F1083-18 – Standard Specification for Pipe, Steel, Hot-Dipped Zinc-Coated (Galvanized) Welded, for Fence Structures.

##### CLFMI-FIG-0111-14 – Field Inspection Guide, CLFMI (Chain Link Fence Manufacturers Institute).

##### CLF-PM0610-17 – Product Manual, CLFMI (Chain Link Fence Manufacturers Institute).

## SYSTEM DESCRIPTION

##### Fence Height: 6 feet nominal.

##### Line Post Spacing: At intervals not exceeding 10 feet.

## SUBMITTALS

##### Section 01 33 00 – Submittal Procedures: Submittal procedures.

##### Shop Drawings:  Indicate plan layout, spacing of components, post foundation dimensions, hardware anchorage, gates, and schedule of components.

##### Product Data:  Submit data on fabric, posts, accessories, fittings and hardware.

## QUALITY ASSURANCE

##### Section 01 40 00 – Quality Requirements.

##### Supply material in accordance with CLFMI – Product Manual.

##### Perform installation in accordance with ASTM F567.

##### Perform quality assurance in accordance with CLFMI-FIG-0111-14 – Field Inspection Guide.

## DELIVERY, STORAGE AND HANDLING

##### Section 01 60 00 – Product Requirements:  Product storage and handling requirements.

##### Fence fabric and accessories shall be delivered to the construction site in packed cartons or firmly tied rolls.

##### Each package shall be identified and shall bear the manufacturer’s name.

##### Store fence fabric and accessories in a secure and dry place.

# PRODUCTS

### MANUFACTURERS

##### Manufacturers:

###### Anchor Fence Inc.

###### Cyclone Inc.

###### Substitutions:  Section 01 60 00 – Product Requirements: or approved equal.

### MATESRIALS

##### Framing (Steel):  ASTM F1083 Schedule 40 galvanized steel pipe, welded construction, minimum yield strength of 25 ksi; coating conforming to ASTM F1043 Type A on pipe exterior and interior.

##### Fabric Wire (Steel):  ASTM A392 zinc coated wire fabric.

##### Concrete:  Section 03 30 00.

### COMPONENTS

##### Terminal, Corner, and Gate Posts:  2.88-inch diameter.

##### Line Posts: 1.9-inch diameter.

##### Top and Brace Rail: 1.66-inch diameter, plain end, sleeve coupled.

##### Gate Frame:

###### 1.9-inch diameter for welded fabrication.

###### Hot-dip galvanize after fabrication.

##### Gusset Plates:  1/4-inch thick steel for truss rod attachment; fabricate to weld into gate frame corners.  Hot-dip galvanize after fabrication.

##### Fabric: 2-inch diamond mesh interwoven wire, 9 gage thick, top salvage and bottom selvage knuckled.

##### Tension Wire: 6 gage thick steel, single strand.

##### Tension Band: Galvanized steel.

##### Tension Strap: Galvanized steel.

##### Tie Wire:  Aluminum alloy steel wire.

### ACCESSORIES

##### Caps:  Dome type, cast steel galvanized; sized to post diameter, set screw retainer.

##### Fittings:  Sleeves, bands, clips, rail ends, tension bars, fasteners and fittings: galvanized steel.

##### Gate Hardware:

###### Fork latch with gravity drop rod and latch assembly.

###### Two 180 degree gate hinges per leaf.

##### Touch-Up Primer for Galvanized Surfaces:  ZRC cold-galvanizing paint, with 94 percent zinc by weight in dried condition.

##### Privacy Slats:  Vinyl strips, sized to fit fabric weave. Color: as selected.

# EXECUTION

#### INSTALLATION

##### Install framework, fabric, and accessories, in accordance with ASTM F567.

##### Place fabric on outside of posts and rails.

##### Set posts plumb, in concrete footings with top of footing 1 inch above finish grade.  Slope top of concrete for water runoff.

##### Do not stretch fabric until concrete foundation has cured.

##### Stretch fabric between terminal posts.

##### Position bottom of fabric 2 inches above finished grade.

##### Fasten fabric to top rail, line posts, braces, and bottom tension wire with tie wire at maximum 18 inches on centers.

##### Attach fabric to posts with tension bars and tension bar clips.

##### Install bottom tension wire stretched taut between terminal posts.

##### Connect to existing fence at Picnic Area by installation of an end post.

##### The clear opening from end posts to buildings, fences and other structures shall not exceed 6 inches unless otherwise approved in advance by the Consultant.

##### Excavate holes for posts to the diameter and spacing required without disturbing the underlying materials.

##### Center and align posts.  Place concrete around posts, and vibrate or tamp for consolidation.

###### Recheck vertical and top alignment of posts and make necessary corrections.

###### Extend concrete footings 1 inch above grade, and trowel to a crown to shed water.

###### Unless otherwise approved by the Consultant, no materials shall be installed on the posts, nor shall the posts be disturbed within 7 days after the individual post footing is completed.

#### ERECTION TOLERANCES

##### Section 01 40 00 – Quality Requirements:  Tolerances.

##### Maximum Variation From Plumb: 1/4 inch.

##### Maximum Offset From True Position: 1 inch.

END OF SECTION